NetAuto Lab — Network Documentation

Cisco Firepower 1010 (FTD) edge firewall + FortiSwitch 108F access switch

Generated 2026-09-12 16:04 · both devices read live from the NetAuto host on VLAN 10 · read-only

# 1  Overview

This document describes a two-device network lab: a Cisco Firepower 1010 running Firepower Threat Defense (FTD) as the edge firewall and inter-VLAN router, and a FortiSwitch 108F running FortiSwitchOS as the access switch. It is the reference network for exercising the netauto toolkit against real, multi-vendor hardware.

The design collapses management onto the data path. Rather than a dedicated out-of-band management network, both devices are administered in-band on VLAN 10 — the users VLAN — with access restricted to a single automation host. This keeps the physical build to two boxes and one cable between them while preserving a clear security boundary: administrative reach is limited to one address, not a whole subnet.

The firewall carries three internal VLANs — users, servers and wifi — each in its own security zone, routed and inspected by the FTD, with wireless isolated from the wired segments in both directions. The switch presents those VLANs to endpoints and trunks them to the firewall on a single 802.1Q link.

# 2  Network topology

Traffic flows vertically: the internet arrives via a cable modem on the firewall's outside interface (Ethernet1/1, DHCP); internal VLANs hang off the switch and reach the internet through the firewall with dynamic PAT. A single 802.1Q trunk between the firewall's Ethernet1/8 and the switch's port1 carries all three VLANs.

Figure 1 — Lab topology. Editable source: docs/netauto-lab-topology.drawio

# 3  Addressing and VLANs

Data VLANs sit inside 10.10.0.0/18; the third octet matches the VLAN ID so an address reads without a lookup. The firewall is the default gateway for every data VLAN. Management shares VLAN 10.

| VLAN | Name | Subnet | Gateway (FTD) | Client addressing |
| --- | --- | --- | --- | --- |
| 10 | Users + management | 10.10.10.0/24 | 10.10.10.1 | DHCP .100–.200 |
| 20 | Servers | 10.10.20.0/24 | 10.10.20.1 | Static |
| 30 | WiFi | 10.10.30.0/24 | 10.10.30.1 | DHCP .100–.200 |
| 1 | Native (trunk) | — | — | carries nothing; untagged frames dropped |

# 4  Cisco Firepower 1010 (FTD) — edge firewall

Model Cisco Firepower 1010 Threat Defense, software FTD 7.0.1-84, hostname ftd-edge-01, routed mode, managed on-box by Firepower Device Manager (FDM). netauto has no FTD driver, so the firewall is not in the inventory and is not audited by the toolkit; its configuration was applied through the FDM REST API and is read back here over that same API.

## 4.1  Interfaces and the hardware switch

The 1010 has a built-in hardware switch. Ethernet1/1 is a routed firewall interface (the WAN); Ethernet1/8 is configured as an 802.1Q trunk; the remaining data ports are disabled. Layer-3 for each internal VLAN is provided by a VLAN interface (SVI).

| Interface | Logical name | State | Role |
| --- | --- | --- | --- |
| Management1/1 | diagnostic | enabled | diagnostic |
| Ethernet1/1 | outside | enabled | outside |
| Ethernet1/3 | — | disabled | — |
| Ethernet1/5 | — | disabled | — |
| Ethernet1/6 | — | disabled | — |
| Ethernet1/7 | — | disabled | — |
| Ethernet1/8 | — | enabled | TRUNK — native VLAN1, tagged wifi,servers,users |
| Ethernet1/4 | — | disabled | — |
| Ethernet1/2 | — | disabled | — |

## 4.2  VLAN interfaces (routing)

| Interface | Name | VLAN | IP address | Zone |
| --- | --- | --- | --- | --- |
| Vlan1 | inside | 1 | 192.168.95.1 | — |
| Vlan20 | servers | 20 | 10.10.20.1 | servers-zone |
| Vlan10 | users | 10 | 10.10.10.1 | users-zone |
| Vlan30 | wifi | 30 | 10.10.30.1 | wifi-zone |

## 4.3  Security zones

Zones are what the access policy matches on — one VLAN per zone, so policy never silently merges two segments.

| Zone | Interfaces |
| --- | --- |
| outside-zone | outside |
| servers-zone | servers |
| users-zone | users |
| wifi-zone | wifi |

## 4.4  Access control policy

The default action is block. Rules evaluate top to bottom, first match wins. The two block rules at the top state WiFi isolation explicitly — redundant against the default deny, but they give named, hit-countable rules for an audit rather than an invisible implicit drop. The permit rules then allow users↔servers and every VLAN out to the internet.

| # | Rule | Action | Source | Destination |
| --- | --- | --- | --- | --- |
| 1 | wifi-to-internal-block | DENY | wifi-zone | servers-zone,users-zone |
| 2 | internal-to-wifi-block | DENY | servers-zone,users-zone | wifi-zone |
| 3 | users-to-servers | PERMIT | users-zone | servers-zone |
| 4 | servers-to-users | PERMIT | servers-zone | users-zone |
| 5 | users-to-internet | PERMIT | users-zone | outside-zone |
| 6 | servers-to-internet | PERMIT | servers-zone | outside-zone |
| 7 | wifi-to-internet | PERMIT | wifi-zone | outside-zone |

## 4.5  Network address translation

Three dynamic PAT rules hide each internal VLAN behind the outside interface address. Because every rule targets the outside interface, inter-VLAN traffic matches none of them and is routed untranslated.

| Rule | Original source | Egress interface | Translation |
| --- | --- | --- | --- |
| pat-users | net-users | outside | dynamic PAT (interface) |
| pat-servers | net-servers | outside | dynamic PAT (interface) |
| pat-wifi | net-wifi | outside | dynamic PAT (interface) |

## 4.6  DHCP services

| Interface | Pool | Enabled |
| --- | --- | --- |
| inside | 192.168.95.5-192.168.95.254 | yes |
| users | 10.10.10.100-10.10.10.200 | yes |
| wifi | 10.10.30.100-10.10.30.200 | yes |

Clients receive DNS 8.8.8.8 directly, so the firewall is not a resolver for the data VLANs.

## 4.7  Management access

FDM is reachable two ways. The dedicated Management1/1 port (10.10.99.10) is the out-of-band path, normally left unplugged. In-band, FDM answers on the VLAN 10 interface, restricted to the automation host:

| Interface | Protocols | Allowed sources |
| --- | --- | --- |
| users | HTTPS, SSH | host-netauto |
| inside | HTTPS | any-ipv4, any-ipv6 |

# 5  FortiSwitch 108F — access switch

Model FortiSwitch-108F, software FortiSwitchOS 7.2.7, hostname fsw-access-01, serial (redacted), standalone mode. Managed by netauto as inventory device fsw-access-01 and reached over SSH at 10.10.10.12; its running configuration and compliance audit below are read live by the toolkit.

## 5.1  VLANs

| VLAN | Description |
| --- | --- |
| 10 | Users and management |
| 20 | Servers |
| 30 | WiFi |

## 5.2  Ports and roles

port1 is the trunk uplink to the firewall. port3 and port5 are user access ports on VLAN 10; port6 is a server on VLAN 20; port4 trunks the access point (VLAN 30 for the SSID, VLAN 10 for management). Access ports enable BPDU guard, since an end host should never send spanning-tree BPDUs. Ports 2, 7 and 8 are administratively down; port9 and port10 are the two SFP uplink ports, currently unused.

| Port | Type | Description | Role | BPDU guard |
| --- | --- | --- | --- | --- |
| port1 | GE | Trunk to FTD Ethernet1/8 | trunk (allowed 10,20,30) | — |
| port2 | GE | — | disabled | — |
| port3 | GE | — | access VLAN 10 | yes |
| port4 | GE | — | trunk (allowed 30) | — |
| port5 | GE | — | access VLAN 10 | yes |
| port6 | GE | — | access VLAN 20 | yes |
| port7 | GE | — | disabled | — |
| port8 | GE | — | disabled | — |
| port9 | SFP uplink | — | access VLAN 1 | — |
| port10 | SFP uplink | — | access VLAN 1 | — |

## 5.3  In-band management interface

The internal management interface holds 10.10.10.12 255.255.255.0 and is bound to VLAN 10 (native-vlan 10), reachable for ping https ssh snmp. The default route points at the firewall's VLAN 10 gateway, 10.10.10.1.

## 5.4  Administrative security, logging and time

Administrative access is restricted to trusted host 10.10.10.20 — a /32, not the VLAN 10 subnet, so a shared management VLAN does not turn every user machine into an administrative source. Logs are sent to syslog at 10.10.10.20, and time is synchronised from NTP servers 216.239.35.0, 216.239.35.4.

## 5.5  Compliance audit (netauto)

netauto's vendor hardening ruleset, evaluated live against the running configuration: 5 pass, 0 fail. A clean result — telnet/HTTP management off, no default SNMP communities, admin idle timeout set, remote logging and a time source configured.

| Rule | Title | Result |
| --- | --- | --- |
| NA-030 | Management interfaces do not allow telnet or HTTP | PASS |
| NA-100 | No default SNMP community strings | PASS |
| NA-102 | Logs are sent off the device | PASS |
| NA-031 | Admin idle timeout is configured | PASS |
| NA-101 | A time source is configured | PASS |

# 6  Management and automation

The netauto host on VLAN 10 (10.10.10.20) reaches both devices from one position: the FortiSwitch over SSH and the firewall over the FDM REST API in-band. The FortiSwitch is a first-class netauto inventory device — discoverable, auditable, and documentable by the toolkit's own workflows. The firewall, lacking a driver, is configured and read through FDM's API directly.

# 7  Verification and current status

Both devices were read live for this document from the VLAN 10 management position. The FortiSwitch audits clean. The firewall's data plane is deployed with zero pending changes.

Not yet verified end to end: inter-VLAN routing and client DHCP leases, which need a test client on a data port. Internet egress waits on the WAN link — Ethernet1/1 is configured for DHCP but the modem link is not yet up.

## Appendix — how this was collected

FortiSwitch: netauto Network Documentation Maker and direct show commands over SSH (fortinet_cli driver). Firepower: FDM REST API (/api/fdm/latest) read in-band at https://10.10.10.1. Diagram: docs/netauto-lab-topology.drawio, editable in draw.io. No device was changed in producing this document.